GPS ROUTES - Creating NEW Routes

Before you start:

Assuming you have a GPS (instruction here based on Garmin GPS's), a connecting cable attached to your PC and TrackMaker GPS software. If not, then view INSTRUCTIONS #1 to get started.

Overview:

The GPS can be used as the input device, the connecting cable is the means to transmit the inputted Route to the PC and TrackMaker software is used to manipulate the information and to save it as a ***.gtm file which inturn can then be downloaded into the Users GPS.

Input methods:

(1)(i) GPS - 'key-in' method

(a) Switch on the GPS, through the menu and via SYSTEM SETUP set the operating MODE to SIMULATOR - this is to save batteries as it won't try to find satellites. Ensure the Map Datum is set to Ordnance Survey GB.

(b) Select WAYPOINT from the menu and using a combination of the GPS's Direction Arrows and <ENTER> buttons enter the the first Waypoints name as say, W1 (or START), then from an O.S. map input the first lanes coordinates, eg if co-ords are SE 134727 then input as ..

SE 13400

'BNG' 72700

(c) When done, select NEW and enter the lanes next Waypoint (this may be at the end or possibly another point along the lane if it is long, not straight or has junctions along its length), name this W2.

(d) Repeat (c) until a maximum of 30 Waypoints are entered, remember to include Waypoints for the 'tarmac' sections inbetween the greenlanes so that a full continuous Route is created.

(e) When all the Waypoints are completed, select INTERFACE menu and set to GRMN/GRMN, plug the lead from the GPS to PC.

(f) Open TrackMaker, select INTERFACE then GARMIN, press PRODUCT ID then REQUEST FROM GPS, press ALL and all Waypoints will be uploaded to TrackMaker. Go to 'TrackMaker - Creating a Route'

(ii) GPS - 'log track' enroute

(a) Switch on the GPS, through the menu and via SYSTEM SETUP set the operating MODE to NORMAL. Ensure the Map Datum is set to Ordnance Survey GB.

(b) Using a combination of the GPS's Direction Arrows and <ENTER> buttons press TRACK LOG from the GPS menu, set RECORD to 'Yes', set TIME INTERVAL to say '30 seconds'.

(c) Enroute, up to 768 track points will automatically be logged, compliment this by logging Waypoints manually at prominent locations (ie corners, junctions etc) by pressing the MARK button - this will log a Waypoint at the current location.

(d) Follow (e) above.

(2) TrackMaker - import a map

(a) Using a scanner, create a .jpg file (at 100 pixels/inch) of a section of the O.S. map that contains all the greenlanes that will make a Route.

(b) In TrackMaker, press TOOLS then OPTIONS, select 'DATUM' as 'OSGB1936 <England>' and 'COORDINATES' as 'Deg/Mins'.

(c) Press TOOLS then INSERT MAP IMAGE to import the scanned image.

(d) Click on the MAP ADJUST TOOL icon - this is to set the scale of the imported map. Position the feint blue cross #1 in a top right position and enter the Longitude (left to right) and Lattitude (top to bottom) in the boxes followed by #2 at the bottom left on the map. The Long./Lat. format, for example, is  ..

LAT.      53 14.0'

LONG.   -3 19.0'

This is Latitude 53 degrees 14 minutes North and Longitude 3 degrees 19 minutes West (note the 'minus' sign !)

The latitude and longitude values can be found on the edges of O.S. maps - with various reference points (as feint blue crosses) on the map. It is important this is done accurately !

(e) Click the PENCIL icon on (A on screenshot below), and using the left mouse button - click to select the Waypoints directly on screen, the 'name box' can be used to input a Waypoint name (ie: W1, W2 etc).Go to 'TrackMaker - Creating a Route'

TrackMaker - Creating a Route:

Whether Waypoints have been inputted from the GPS or inputted on screen then the next job is to join them together in numerical order (Waypoint order) to create a Route. Click the PENCIL (A above) and the SNAP icons (B) on, press and hold the keyboard <SHIFT> key on, and one after the other left-click to join the Waypoints together - one at a time. If you do one wrong, switch A off - click the incorrect section of Route, then press the <DELETE> key.

If you break off from creating the Route then you may find the Route has changed colour (from blue), this means you have started a 2nd Route. Delete as described above and redo it - but join the 'latter' Waypoint to the 'former' (ie: in reverse), this will recomence the original (blue) Route.

Waypoints uploaded from the GPS will have 'date & time' information that will clutter the screen, this can be turned off via the SHOW (C) icon, or better still right-click (A off, B on) and clear the 'description' box.

Waypoints can be turned on and off enmass using the INVERT SELECTED POINTS (E) icon. When all the Waypoints are selected and the Route is complete then its length can be determined by pressing the CALCULATE LENGTH (D) icon.

The Route can be given a name by right-clicking on the Route and entering a name in the 'Route name' dialogue box.

If the Route has been inputted on screen, then before saving the Route the map image needs to be deleted via TOOLS then MAP IMAGE PROPERTIES.

TrackMaker - Loading the Route back into the GPS:

Through the GPS's menu go to INTERFACE, using the GPS's Direction Arrows and <ENTER> buttons and ensure the Interface is set to GRMN/GRMN (as opposed to NONE/NONE or anything else).

Through TrackMaker's menu select INTERFACE>GARMIN INTERFACE, in the pop-up box press PRODUCT ID, if everything is plugged in and correctly connected then the name & model number of the GPS will be displayed in the left window. Press SEND TO GPS then press ALL - the Route will now download.

The Route in the GPS:

Through the GPS's menu go to ROUTES, Route #1 will contain the Route just downloaded,  save it for the future using the GPS's COPY feature - save it under a different Route #, as #1 is usually used to download to.

The Routes name will be shown near to the top of the screen, press ACT the activate the route, this automatically moves the Route to #0 (which is always the currently active route). Consult your GPS's instruction manual for more details on how to follow the Route on the ground, I personally prefer the Moving Map mode where an arrow guides you to each successive waypoint. OSGB co-ordinates will be available on screen if required - again consult your manual.

Note:

If you do create any of your own Route (***.gtm) files, please email them to me at northernlanes@freeuk.com and I will include them on the Northern Greenlanes website www.northernlanes.freeuk.com